EASY RIDE - Specifications

1.	Creation of a permeable foundation layer for the plastic pavement, type Easy Ride , through the supply and positioning of small sized crushed stone, variable grain of 3/10 mm, spread out and rolled for a finished thickness of 15-20 cm; possible bedding layer with siliceous sand, properly levelled, approx. 4 cm. Laying of the geotextile prior to the development of the flooring.

By the square meter e_______________________________________

2.	Supply and installation of the Easy Ride pavement from the Daliform Group Srl with the following characteristics:

•	100% recycled material;

•	Load capacity no less than 200 T/m2 (monotonic rupture load test  );

•	Male female interlocking hooking system and claw retainers;

The newly created pavement will have the technical features for stabilising the surfaces covered by the animals, also large ones, preserving mobility and safeguarding general hygiene.

By the square meter e_______________________________________

3.	Level filling of the cells with volcanic lapillus (variable grain 0-4 mm) to keep the foundation moist, thanks to its porosity, preventing problems with dust. Then, apply a 20 cm layer of silica sand for levelling purposes.

By the square meter e_______________________________________

The pavement must be produced by a Certified Company with an Integrated Management System UNI EN ISO 9001 (Quality), UNI EN ISO 14001 (Environment); UNI EN ISO 45001 (Safety) and SA 8000 (Social Responsibility).