Specifications

Extensive roof-top gardens

Supply and installation of anti-root sheath. For extra protection of the waterproofing it is recommended to lay a piece of geotextile (TNT = Non-Woven Fabric) of 200 g/m2.

Supply and installation of Iglu'® green roof with tank made in "ALAPLEN® CV30", resistant to organic substances, which acts as a collection and draining layer.

Filling of Iglu'® green roof with tank to a depth of 2 cm above the edge, with pumice or volcanic lapilli, particle size 10-12 mm, high level of water absorption.

Laying of 150 g/m2 geotextile (TNT = Non-Woven Fabric) as a filter layer of separation between the filling material and the substrate.

Substrate to a thickness varying between 8 and 20 cm.

Realisation of an irrigation system, if any.

Intensive green roof

Supply and installation of anti-root sheath. For extra protection of the waterproofing it is recommended to lay a piece of Geotextile (TNT = Non-Woven Fabric) of 200 g/m2.

Supply and installation of Iglu'® green roof with dispersion made in "ALAPLEN® CV30", resistant to organic substances, which acts as a collection and draining layer.

Filling of Iglu'® green roof with dispersion to a depth of 2 cm above the edge, with pumice or volcanic lapilli, particle size 10-12 mm, high level of water absorption.

Laying of 150 g/m2 geotextile (TNT = Non-Woven Fabric) as a filter layer of separation between the filling material and the substrate.

Substrate to a thickness varying between 20 e 150 cm.

Implementation of an ad hoc irrigation system.