EOLO - Specifications

Implementation of a ventilated under-ground cavity with the supply and on-site positioning of recycled plastic Eoloformworks from the Daliform Group for the quick dry formation of a self-bearing pedestrian accessible platform above which the concrete (minimum resistance class C25/30) is cast to fill the formwork up to its crown and an upper slab of ______ cm reinforced with rebars of diameter Ø ______ cm ___ x ___ cm, levelled and smoothed with a plastering trowel.

Eolo formworks must have dimensions of 50 x 50 cm (centre distance) and 27 cm in height, with a convex shape to be placed only on the four lateral feet to guarantee maximum ventilation and 6 cm high nozzles to guarantee the realisation of perforated floors for the distribution of air in composting, waste stabilisation systems, deodorising systems and food storage rooms.

Eolo formworks provide breaking resistance of 150 kg in correspondence of the centre of the arch with an 8 x 8 cm clamp.

Formworks in recycle plastic, such as EOLO must be produced in "ALAPLEN® CP30", must not release polluting substances, have an Environmental Compatibility Certification and be produced by a Company Certified according to International Standards UNI EN ISO 9001 (Quality), UNI EN ISO 14001 (Environment); UNI EN ISO 45001 (Safety) and SA 8000 (Social responsibility).

The company that supplies the EOLO formworks must exhibit technical sheet and safety sheet for the product and for "ALAPLEN® CP30" grain texture and must also exhibit the product certificate approved by an EOTA member agency (European Organisation for Technical Approvals).

By the square meter €_______________________________________